## Name: _______________________ Rank: ____Unit: _____________________

## Weapon System: ________DRS___________ TWS Version: _____(1)_______

## PERFORMANCE MEASURES (Phase 1)

Place LTWS into operation.

GO N/G

|  |  | - Inventoried TWS components per TM. |
| --- | --- | --- |
|  |  | - Check the housing of the TWS for cracks, dents and dirt. |
|  |  | - Ensure the eyecup and lens cap are present with no damage. |
|  |  | - Inspect the control pad for cracks, dry rot and dirt. |
|  |  | - Ensure objective focus ring turns smoothly to the end of travel. |
|  |  | - Ensure that the purge screw is present and not loose. |
|  |  | - Ensure that the I/O port cap and lanyard are present, remove the cap and inspect for broken pins inside of the I/O port . |
|  |  | - Inspect the lenses for cracks, scratches and dirt, (if objective lens is damaged, do not ingest or inhale the crystals). |
|  |  | - Remove battery cassette from TWS and inspect for damage, corrosion and inside of the battery compartment. Inspect the spare battery cassette. |
|  |  | - Install batteries into battery cassette and install cassette into LWTS. |
|  |  | - Remove carrying handle and mount appropriate bracket if applicable. |
|  |  | - Loosen the torque limiting knob on the rail grabber. |
|  |  | - Place LTWS on the M16A1/A2/A4 or M4. |
|  |  | - Place LTWS rail grabber completely on the picatinny rail. |
|  |  | - Mount the LTWS to the weapon. Tighten down the torque limiting knob, and hear two audible clicks while applying forward pressure. |
|  |  | - Turn TWS on. Wait for the start up period to end. Ensure there are no failures. |
|  |  | - Adjusts the brightness of the screen to the individuals eye. |
|  |  | - Press the FOV/Mode button for 3 seconds to get into the menu. |
|  |  | - Change TWS to I/O out. |
|  |  | - Select the RET that coincides w/ your weapon system. |
|  |  | - Turn on the Stadiametric Scales. |
|  |  | - Selects RET POL WHT/BLK/ARC. |
|  |  | - Uncover the objective lens. |
|  |  | - Adjust the objective focus ring to obtain a clear scene. |
|  |  | - Perform a shutterless calibration without covering the objective lens. |
|  |  | - Perform a shuttered calibration to clear ghost image. |
|  |  | - Turn AGC off. |
|  |  | - Adjust Manual Level. |
|  |  | - Adjust Manual gain. |
|  |  | - Switch to AGC on. |
|  |  | - Adjust fine level. |
|  |  | - Adjust fine gain. |
|  |  | - Select Polarity WHT HOT/BLK HOT. |
|  |  | - Switch field of view. |
|  |  | - Make any other needed optimization adjustments. |
|  |  | - Reconfigure weapon to clip-on mode. |
|  |  | - Change mode in LWTS to clip-on. |
|  |  | - Change LWTS to I/O in. |
|  |  | - Perform a RESET. |

Take TWS Out of Operation

|  |  | - Turn TWS off and close lens cover. |
| --- | --- | --- |
|  |  | - Loosen the torque-limiting knob. |
|  |  | - Remove the TWS from weapon. |
|  |  | - Remove the bracket (if applicable). |
|  |  | - Tighten torque limiting knob, and torque limiting knob on bracket if applicable. |
|  |  | - Remove battery cassette. |
|  |  | - Install the empty battery cassette into TWS. |
|  |  | - Place full battery cassette into the storage case. |
|  |  | - Properly place the TWS into the carrying case. |
|  |  | 49. Places all BII into the carrying case. |