BFV Safety

071F0701 / Version A

01 Oct 2006

SECTION I.	ADMINISTRATIVE DATA

| All Courses Including This Lesson | Course Number Version Course Title 071-F8 A Bradley Crew Evaluator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task(s) Taught(*) or Supported | Task Number Task Title |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reinforced Task(s) | Task Number Task Title |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Academic Hours | The academic hours required to teach this lesson are as follows: Resident Hours/Methods 2 hrs 10 mins / Conference / Discussion Test 0 hrs Test Review 0 hrs Total Hours: 2 hrs 10 mins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Lesson Number | Hours Lesson No. Testing (to include test review) N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prerequisite Lesson(s) | Lesson Number Lesson Title None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clearance Access | Security Level: Unclassified Requirements: There are no clearance or access requirements for the lesson. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Foreign Disclosure Restrictions | FD5. This product/publication has been reviewed by the product developers in coordination with the USAIC foreign disclosure authority. This product is releasable to students from all requesting foreign countries without restrictions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| References | Number | Title |  |  | Date |  |  |  |  |  | Additional Information |  |  |  |  |  |  |
|  | FM 3-22.1 | Bradley Gunnery |  |  | 28 Nov 2003 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FM 5-19 | Composite Risk Management |  |  | 21 Aug 2006 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TC 3-34.489 | The Soldier and the Environment |  |  | 08 May 2001 |  |  |  |  |  |  |  |  |  |  |  |  |
| Student Study Assignments | None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructor Requirements | ASIJ3 Qualified Master Gunner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional Support | Name |  |  |  |  |  |  | Stu Ratio |  | Qty |  | Man Hours |  |  |  |  |  |
| Personnel Requirements | None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Required | Id Name |  |  |  |  | Stu Ratio |  |  | Instr Ratio |  | Spt |  |  |  | Qty |  | Exp |
| for Instruction | 0000-00-0.U42808 STOP WATCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 10 |  | No |
|  | 4470-01-359-2799 COMPUTER, NOTEBOOK/LAPTOP |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 1 |  | No |
|  | 6730-00-577-4813 SCREEN, PROJECTION |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 1 |  | No |
|  | 6730-01-T09-1615 PROJECTOR, LIGHT PRO INFOCUS |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 1 |  | No |
|  | 7520-00-281-5918 CLIPBOARD FILE |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 10 |  | Yes |
|  | * Before Id indicates a TADSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Materials Required | Instructor Materials: None. Student Materials: FM 3-22.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Classroom, Training Area, and Range Requirements | ORGANIZATIONAL CLASSROOM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ammunition Requirements | Id Name |  |  |  |  |  | Exp |  | Stu Ratio |  |  |  | Instr Ratio |  |  | Spt Qty |  |
|  | None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructional Guidance | NOTE: Before presenting this lesson, instructors must thoroughly prepare by studying this lesson and identified reference material. The Primary Instructor for this lesson should prepare well in advance. The instructor should review all references, examine all student materials; examine, and practice using all training aids, devices, simulators, and simulations associated with the conduct of training under this lesson plan before conducting the training. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proponent Lesson Plan Approvals | Name SACG, ARNG, |  | Rank MSG | Position Sr BFV MG |  |  |  |  |  |  |  |  |  | Date 01 Oct 2006 |  |  |  |

SECTION II.	INTRODUCTION

Method of Instruction:  Conference / Discussion

Instructor to Student Ratio is:  1:20

Time of Instruction:  5 mins

Media:  Slides

| Motivator | Show slide #1 Develop locally Show slide #2 TLO Show slide #3 Administrated notes |  |
| --- | --- | --- |
| Terminal Learning Objective | NOTE: Inform the students of the following Terminal Learning Objective requirements. At the completion of this lesson, you [the student] will: |  |
|  | Action: | Identify all areas of safety that apply to the Bradley Fighting Vehicle (BFV). |
|  | Conditions: | In a classroom environment given FM 3-22.1. |
|  | Standards: | Achieve a minimum of 90% on a end of course written examination. |
| Safety Requirements | The Primary Instructor is responsible for the implementation of safety measures according to the local safety Standard Operating Procedures. If appropriate, include a building evacuation plan. |  |
| Risk Assessment Level | Low - The Primary Instructor will prepare a risk assessment using the before, during, and after checklists IAW FM 5-19. It will be prepared prior to the block of instruction, event or activity. |  |
| Environmental Considerations | NOTE: It is the responsibility of all Soldiers and DA civilians to protect the environment from damage. The Primary Instructor will prepare an environmental risk assessment using The Soldier and The Environment Training Circular (TC 3-34.489). The checklist should be supplemented with state and local environmental regulations that apply to your installation. Instructors should ensure that trash is removed from the classroom at the end of the day or as needed. |  |
| Evaluation | Student must receive GO on a hands-on evaluation and 90% or better on an End of Course exam. |  |
| Instructional Lead-In | Develop locally. Show slide #4 ELO A |  |

SECTION III.	PRESENTATION

NOTE:	Inform the students of the Enabling Learning Objective requirements.

A.	ENABLING LEARNING OBJECTIVE

| ACTION: | Identify general BFV safety procedures. |
| --- | --- |
| CONDITIONS: | In a classroom environment, given FM 3-22.1. |
| STANDARDS: | Identify general BFV safety procedures, IAW FM 3-22.1, Ch 5. |

1.	Learning Step / Activity 1. Identify general BFV safety procedures

Method of Instruction:	Conference / Discussion

Instructor to Student Ratio:	1:20

Time of Instruction:	20 mins

Media:	Slides

Show slide #5

General Safety

No smoking within 50 feet of the vehicle

No “horse playing” around the vehicle

Observe all caution and warning signs

Maintain 3 points of contact while on top of the vehicle

No sleeping on, under, or around a BFV

CHECK ON LEARNING:  Conduct a check on learning to summarize the ELO.

Show slide #6

ELO B

B.	ENABLING LEARNING OBJECTIVE

| ACTION: | Identify Vehicle movement safety procedures |
| --- | --- |
| CONDITIONS: | In a classroom environment given FM 3-22.1. |
| STANDARDS: | Identify vehicle movement safety procedures IAW FM 3-22.1, CH 5 |

1.	Learning Step / Activity 1. Identify vehicle movement safety procedures

Method of Instruction:	Conference / Discussion

Instructor to Student Ratio:	1:20

Time of Instruction:	20 mins

Media:	Slides

Show slide #7

Vehicle Movement Safety

Keep hatches locked when in the open (hatch safety pins) or closed position

Honk horn before lowering and raising the ramp only after an observer has cleared the area

Ensure all personnel wear a cvc of kevlar when inside the vehicle

Show slide #8

Vehicle Movement Safety

Ensure communication is maintained between bc and driver

Always use two ground guides when moving the vehicle in close spaces.

CHECK ON LEARNING:  Conduct a check on learning to summarize the ELO.

Show slide #9

ELO C

C.	ENABLING LEARNING OBJECTIVE

| ACTION: | Identify BFV turret safety procedures |
| --- | --- |
| CONDITIONS: | In a classroom environment given FM 3-22.1. |
| STANDARDS: | Identify BFV turret safety procedures IAW, FM 3-22.1, Ch 5 |

1.	Learning Step / Activity 1. Identify BFV turret safety procedures

Method of Instruction:	Conference / Discussion

Instructor to Student Ratio:	1:20

Time of Instruction:	20 mins

Media:	Slides

Show slide #10

Turret Safety

Ensure deck is clear and turret shield door is closed before turning the turret on or

moving the turret Sound off with “traversing”  before moving the turret

Ensure turret travel lock is engaged when the turret is not in operation

Show slide #11

Turret Safety

Personnel should ensure turret drive lights on the turret step are not illuminated

before entering the turret The commander should maintain no more than “name tag”

height when in the open hatch mode.

CHECK ON LEARNING:  Conduct a check on learning to summarize the ELO.

Show slide #12

ELO D

D.	ENABLING LEARNING OBJECTIVE

| ACTION: | Identify BFV range safety procedures |
| --- | --- |
| CONDITIONS: | In a classroom environment given FM 3-22.1. |
| STANDARDS: | Identify BFV range safety procedures, IAW FM 3-22.1, Ch 5 |

1.	Learning Step / Activity 1. Identify BFV range safety procedures

Method of Instruction:	Conference / Discussion

Instructor to Student Ratio:	1:20

Time of Instruction:	20 mins

Media:	Slides

Show slide #13

Range Safety

I.A.W LOCAL RANGE SAFETY SOPS

CHECK ON LEARNING:  Conduct a check on learning to summarize the ELO.

Show slide #14

ELO E

E.	ENABLING LEARNING OBJECTIVE

| ACTION: | Identify range flag colors and meanings used on a BFV. |
| --- | --- |
| CONDITIONS: | In a classroom environment given FM 3-22.1. |
| STANDARDS: | Identify range flag colors and meanings used on a BFV, IAW FM 3-22.1, Ch 5 |

1.	Learning Step / Activity 1. Identify range flag colors and meanings used on a BFV

Method of Instruction:	Conference / Discussion

Instructor to Student Ratio:	1:20

Time of Instruction:	20 mins

Media:	Slides

Show slide #15

BFV range flags

Show slide #16

Red Flag

The Bradley is engaging in firing

The weapon systems are loaded and pointed in the target area

The weapon systems are off of electrical and manual safe

Show slide #17

Green Flag

All weapons systems are cleared and elevated

The electrical and manual safeties are on

All ammunition on the vehicle is either loaded in the ready boxes or stowed

Show slide #18

Yellow Flag

Never flown alone

Show slide #19

Green and Red

The Bradley is preparing to fire or the crew is conducting a non-firing exercise

The 25mm gun feeder may be loaded but the bolt is in sear, and the electrical and

manual safeties are on The coax machine gun may be loaded, but the bolt is to the

rear and the manual safety is on

Show slide #20

Green and Yellow

There is a malfunction on the vehicle and all weapons systems are clear

Show slide #21

Red and Yellow

There is a malfunction or misfire on the vehicle

The weapons systems are pointing at the target area and are not clear

The electrical and manual safeties are on (if the crew cannot set the electrical and

manual safeties they should notify range personnel).

CHECK ON LEARNING:  Conduct a check on learning to summarize the ELO.

SECTION IV.	SUMMARY

Method of Instruction:    Conference / Discussion

Instructor to Student Ratio is:    1:20

Time of Instruction:   5 mins

Media:    Lecture

| Check on Learning | Show slide #23 Question: What flag is never flown alone? Answer: Yellow Question: Range safety Answer: IAW local range SOP Question: Soldiers should always maintain___ points of contact when on top of the vehicle Answer: 3 |
| --- | --- |
| Review / Summarize Lesson | Show slide #22 Summary Action: Identify all areas of safety that apply to the Bradley Fighting Vehicle (BFV) The four areas of BFV safety: General safety Vehicle movement safety Turret safety Range safety Range flags: Red Yellow Green |

SECTION V.	STUDENT EVALUATION

| Testing Requirements | NOTE: Describe how the student must demonstrate accomplishment of the TLO. Refer student to the Student Evaluation Plan. |
| --- | --- |
| Feedback Requirements | NOTE: Feedback is essential to effective learning. Schedule and provide feedback on the evaluation and any information to help answer students' questions about the test. Provide remedial training as needed. |
|  | None. |

Appendix A - Viewgraph Masters  (N/A)

Appendix B - Test(s) and Test Solution(s)  (N/A)

Appendix C - Practical Exercises and Solutions  (N/A)

Appendix D - Student Handouts  (N/A)