BFV Fire Commands

071F0705 / Version A

01 Oct 2006

SECTION I.	ADMINISTRATIVE DATA

| All Courses Including This Lesson | Course Number Version Course Title 071-F8 A Bradley Crew Evaluator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task(s) Taught(*) or Supported | Task Number Task Title |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reinforced Task(s) | Task Number Task Title |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Academic Hours | The academic hours required to teach this lesson are as follows: Resident Hours/Methods 7 hrs 5 mins / Conference / Discussion Test 0 hrs Test Review 0 hrs Total Hours: 7 hrs 5 mins |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Lesson Number | Hours Lesson No. Testing (to include test review) N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prerequisite Lesson(s) | Lesson Number Lesson Title None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clearance Access | Security Level: Unclassified Requirements: There are no clearance or access requirements for the lesson. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Foreign Disclosure Restrictions | FD5. This product/publication has been reviewed by the product developers in coordination with the USAIC foreign disclosure authority. This product is releasable to students from all requesting foreign countries without restrictions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| References | Number | Title |  |  | Date |  |  |  |  |  | Additional Information |  |  |  |  |  |  |
|  | FM 3-22.1 | Bradley Gunnery |  |  | 28 Nov 2003 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FM 5-19 | Composite Risk Management |  |  | 21 Aug 2006 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TC 3-34.489 | The Soldier and the Environment |  |  | 08 May 2001 |  |  |  |  |  |  |  |  |  |  |  |  |
| Student Study Assignments | None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructor Requirements | ASIJ3 Qualified Master Gunner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional Support | Name |  |  |  |  |  |  | Stu Ratio |  | Qty |  | Man Hours |  |  |  |  |  |
| Personnel Requirements | None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Required | Id Name |  |  |  |  | Stu Ratio |  |  | Instr Ratio |  | Spt |  |  |  | Qty |  | Exp |
| for Instruction | 0000-00-0.U42808 STOP WATCH |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 10 |  | No |
|  | 4470-01-359-2799 COMPUTER, NOTEBOOK/LAPTOP |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 1 |  | No |
|  | 6730-00-577-4813 SCREEN, PROJECTION |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 1 |  | No |
|  | 6730-01-T09-1615 PROJECTOR, LIGHT PRO INFOCUS |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 1 |  | No |
|  | 7520-00-281-5918 CLIPBOARD FILE |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 10 |  | Yes |
|  | * Before Id indicates a TADSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Materials Required | Instructor Materials: None Student Materials: FM 3-22.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Classroom, Training Area, and Range Requirements | ORGANIZATIONAL CLASSROOM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ammunition Requirements | Id Name |  |  |  |  |  | Exp |  | Stu Ratio |  |  |  | Instr Ratio |  |  | Spt Qty |  |
|  | None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instructional Guidance | NOTE: Before presenting this lesson, instructors must thoroughly prepare by studying this lesson and identified reference material. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proponent Lesson Plan Approvals | Name SACG, ARNG, |  | Rank MSG | Position Sr. BFV MG |  |  |  |  |  |  |  |  |  | Date 01 Oct 2006 |  |  |  |

SECTION II.	INTRODUCTION

Method of Instruction:  Conference / Discussion

Instructor to Student Ratio is:  1:20

Time of Instruction:  4 hrs

Media:  Lecture

| Motivator | Show slide #1 Develop Locally Show slide #2 TLO Show slide #3 Administrative Notes. |  |
| --- | --- | --- |
| Terminal Learning Objective | NOTE: Inform the students of the following Terminal Learning Objective requirements. At the completion of this lesson, you [the student] will: |  |
|  | Action: | Identify BFV fire command procedures. |
|  | Conditions: | In a classroom environment given FM 3-22.1. |
|  | Standards: | Achieve a minimum of 90% on a end of course written examination. |
| Safety Requirements | The Primary Instructor is responsible for the implementation of safety measures according to the local safety Standard Operating Procedures. If appropriate, include a building evacuation plan. |  |
| Risk Assessment Level | Low - The Primary Instructor will prepare an environmental risk assessment using The Soldier and The Environment Training Circular (TC 3-34.489). The checklist should be supplemented with state and local environmental regulations that apply to your installation. Instructors should ensure that trash is removed from the classroom at the end of the day or as needed. |  |
| Environmental Considerations | NOTE: It is the responsibility of all Soldiers and DA civilians to protect the environment from damage. The Primary Instructor will prepare an environmental risk assessment using The Soldier and The Environment Training Circular (TC 3-34.489). The checklist should be supplemented with state and local environmental regulations that apply to your installation. Instructors should ensure that trash is removed from the classroom at the end of the day or as needed. |  |
| Evaluation | Student must receive a GO on hands-on evaluation and 90% or better on the End of Course exam. |  |
| Instructional Lead-In | Develop Locally. |  |

SECTION III.	PRESENTATION

NOTE:	Inform the students of the Enabling Learning Objective requirements.

A.	ENABLING LEARNING OBJECTIVE

| ACTION: | Define fire commands, and identify the elements of fire commands. |
| --- | --- |
| CONDITIONS: | In a classroom environment given FM 3-22.1. |
| STANDARDS: | Define fire commands, and identify the elements of fire commands, IAW FM 3-22.1, Ch 6. |

1.	Learning Step / Activity 1. Define fire commands, and identify the elements of fire commands.

Method of Instruction:	Conference / Discussion

Instructor to Student Ratio:	1:20

Time of Instruction:	3 hrs

Media:	Slides

Show slide #5

Fire Commands

Fire commands are the language of gunnery. Every Direct-fire

engagement begins with a fire command. They can only be given

after target presentation during preliminary and intermediate BFV

tables.

Show slide #6

Fire Commands Elements

Alert

Weapons/Ammuntion

Description

Direction (optional)

Range (optional on ODS)

Execution

Termination

Show slide #7

Alert Terms

Gunner

Turret weapon system

Rear bank

Firing port weapons

Show slide #8

Wpns/Ammo Terms

WEAPON AND AMMUNITION	ANNOUNCED AS (TERM)

25-MM HEI-T	“HE” AITCH –E

25-MM APDS-T, APFSDS-T	“SABOT” SAYBO

25-MM TP-T 	“HE” OR “SABOT”

M240C COAX	“COAX”

TOW	“MISSILE”

WHEN FIRING WITH ONLY TP-T OR TPDS-T, ANNOUNCE THE TYPE OF AMMUNITION THE TP-T OR TPDS-T IS REPLICATING (SABOT OR HE)

Show slide #9

Description Terms

TYPE OF TARGET	                            ANNOUNCED AS (TERM)—

TANK 	                                        "TANK"

IFV OR APC	                                        "PC"

UNARMORED VEHICLE	               "TRUCK"

HELICOPTER	                                        "CHOPPER"

FIXED WING AIRCRAFT	               "PLANE, " "JET, " or "UAV" (ADA)

PERSONNEL	                                        "TROOPS"

MACHINE GUN	                                        "MACHINE GUN"

ATGM	                                        "ANTITANK"

ANY OTHER TARGET                              (BRIEFEST TERM POSSIBLE)

Show slide #10

Direction Terms

Traverse

Shift

Reference point and deflection

Range card data

THESE ARE USED TO GUIDE THE GUNNERS AIM TO THE TARGET AREA

Show slide #11

Range Terms

Range to target	Announced as (term)

800 m	            "eight hundred" or "eight"

1,200 m	            "one two hundred," "twelve," or "twelve hundred"

3,040 m                         "three thousand" or "thirty"

If the gunner uses the BELRF on an M2A2 ODS vehicle, it is not necessary for the bc to announce range.

Show slide #12

Execution Terms

Fire

At my command

From my position on the way

Show slide #13

Termination Terms

Cease fire

Terminates 25mm and coax engagements

Cease tracking

Terminates tow missile engagements

Show slide #14

Response Terms

Identified

Cannot identify

Cannot engage

On the way

Target destroyed

The commander or gunner uses these terms in response to a fire command

Show slide #15

Repeat Terms

Crew member fails to hear or understand a part of the fire command he

will repeat the element in question

Correction terms

To correct an error in a fire command, the BC announces correction and

corrects on the element in error

Re-engage

Only used when engaging the same target with the same ammo without

switching either

Show slide #16

Precision Fire Commands M2A2

Used in the defense

Gives the crew time to lay the weapon on target,

Determine accurate range, issue the fire

Command, and adjust the fire control system

Time gives the crew increased probability of a

First round hit

Show slide #17

Precision Fire Command M2A2

ELEMENT                                               COMMANDER                  GUNNER

ALERT                                                        GUNNER

WEAPON/AMMUNITION		  SABOT

DESCRIPTION				  PC

DIRECTION (OPTIONAL)

RANGE			               1600

IDENTIFIED

EXECUTION				  FIRE

ON THE WAY

TERMINATION                                         CEASE FIRE

Show slide #18

Precision Fire Command M2A2

ELEMENT                                               COMMANDER                  GUNNER

ALERT                                                        GUNNER

WEAPON/AMMUNITION		  COAX

DESCRIPTION				  TROOPS

DIRECTION (OPTIONAL)

RANGE			               600

IDENTIFIED

EXECUTION				  FIRE

ON THE WAY

TERMINATION                                          CEASE FIRE

Show slide #19

Precision Fire Command M2A2

ELEMENT                                               COMMANDER                   GUNNER

ALERT                                                        GUNNER

WEAPON/AMMUNITION		  SABOT

DESCRIPTION				  PC

DIRECTION (OPTIONAL)

RANGE			               RANGE

IDENTIFIED

1400

EXECUTION				  FIRE

ON THE WAY

TERMINATION				  CEASE FIRE

Show slide #20

Precision Fire Command ODS

Used in the defense and offense

Less time is needed to lay the weapon system due

To the addition of the BELRF

Show slide #21

Precision Fire Command ODS

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		       SABOT

DESCRIPTION				        PC

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				          FIRE

ON THE WAY

TERMINATION				         CEASE FIRE

The gunners response of identified informs the BC that

the gunner has received a range and discriminated the

target

Show slide #22

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		        SABOT

DESCRIPTION				        PC

DIRECTION (OPTIONAL)

RANGE					                                  RANGE

1200

IDENTIFIED

EXECUTION				          FIRE

ON THE WAY

TERMINATION				          CEASE FIRE

The gunner did not receive a reliable range,so

he announced range to get a range from the BC

Show slide #23

Precision Fire Command ODS

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		        SABOT

DESCRIPTION				        PC

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				  		                       FRIEND

TERMINATION				         CEASE FIRE

Show slide #24

Battlesight Fire Command

USED ON THE M2A2 IN THE OFFENSE WHEN SPEED IS A

NECESSITY

USED ON THE ODS ONLY IN THE EVENT OF A BELRF

FAILURE

THE WORD BATTLESIGHT REPLACES BOTH WEAPON AND

AMMUNITION

THE TARGET DESCRIPTION IDENTIFIES WHICH BATTLE

CARRY AMMUNITION AND RANGE TO USE

EXAMPLE

PC =  SABOT, 1200 M

TRUCK = HE, 800 M

TROOPS = COAX, 600 M :

Show slide #25

Battlesight Fire Command

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		        BATTLESIGHT

DESCRIPTION				        PC

DIRECTION (OPTIONAL)                                TRAVERSE RIGHT

RANGE

IDENTIFIED

EXECUTION				         FIRE

ON THE WAY

TERMINATION				         CEASE FIRE

Show slide #26

Battlesight Fire Command

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		        BATTLESIGHT

DESCRIPTION				        TROOPS

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				           FIRE

ON THE WAY

TERMINATION				          CEASE FIRE

Show slide #27

Subsequent Fire Command

BC CORRECTS GUNNER FIRING BASED ON

OBSERVATIONS

ALERT

BC ANNOUNCES OBSERVATIONS AS THE ALERT

SHORT, OVER)

BC INSTRUCTS GUNNER TO ADD OR SUBTRACT

RANGE

BC INSTRUCTS GUNNER TO ADJUST LEFT OR RIGHT

USING MILS OR TARGET FORMS

BC COMPLETES THE COMMAND WITH THE

COMMAND OF EXECUTION

Show slide #28

Tow Fire Command

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		        MISSILE

DESCRIPTION				        TANK

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				          FIRE

ON THE WAY

TERMINATION				         CEASE TRACKING

Show slide #29

Multiple Fire Commands

Allows crew to engage multiple targets quickly

without giving separate precision or battlesight

fire commands

Quick transition from one target to another

Show slide #30

Multiple Battlesight Fire Command

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                            GUNNER

WEAPON/AMMUNITION		       BATTLESIGHT

DESCRIPTION				       TWO PC’S, LEFT PC

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				       FIRE

ON THE WAY

TERMINATION				       CEASE FIRE

DESCRIPTION                                                SHIFT PC

IDENTIFIED

EXECUTION                                                    FIRE

ON THE WAY

TERMINATION                                                CEASE FIRE

Show slide #31

Multiple Battlesight Fire Command

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		       BATTLESIGHT

DESCRIPTION				       TROOPS, PC, TROOPS

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				       FIRE

ON THE WAY

TERMINATION				       CEASE FIRE

DESCRIPTION                                                SHIFT PC

IDENTIFIED

EXECUTION                                                   FIRE

ON THE WAY

TERMINATION                                               CEASE FIRE

Show slide #32

Multiple Precision Fire Command M2A2

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                             GUNNER

WEAPON/AMMUNITION		        SABOT

DESCRIPTION		                                  TWO PC’S, LEFT PC

DIRECTION (OPTIONAL)

RANGE	                                               1200M

IDENTIFIED

EXECUTION				        FIRE

ON THE WAY

TERMINATION				       CEASE FIRE

DESCRIPTION                                                SHIFT RIGHT SABO PC                         						                     1600M                             IDENTIFIED

EXECUTION                                                    FIRE

ON THE WAY

TERMINATION                                               CEASE FIRE

Show slide #33

Multiple Precision Fire Command M2A2

ELEMENT                                                       COMMANDER                 GUNNER

ALERT                                                              GUNNER

WEAPON/AMMUNITION		        SABOT

DESCRIPTION		                                  TWO PC’S, LEFT PC

DIRECTION (OPTIONAL)

RANGE

IDENTIFIED

EXECUTION				         FIRE

ON THE WAY

TERMINATION				        CEASE FIRE

DESCRIPTION                                                 SHIFT RIGHT SABO PC

IDENTIFIED

EXECUTION                                                     FIRE

ON THE WAY

TERMINATION                                                 CEASE FIRE

CHECK ON LEARNING:  Conduct a check on learning and summarize the ELO.

SECTION IV.	SUMMARY

Method of Instruction:    Conference / Discussion

Instructor to Student Ratio is:    1:20

Time of Instruction:   5 mins

Media:    Lecture

| Check on Learning | Show slide #35 Check on Learning Devlop Locally. |
| --- | --- |
| Review / Summarize Lesson | Show slide #34 Summary Learn how to properly evaluate a BFV crew in device and crew-live fire gunnery. Define fire commands, and identify the elements of fire commands. |

SECTION V.	STUDENT EVALUATION

| Testing Requirements | NOTE: Describe how the student must demonstrate accomplishment of the TLO. Refer student to the Student Evaluation Plan. |
| --- | --- |
| Feedback Requirements | NOTE: Feedback is essential to effective learning. Schedule and provide feedback on the evaluation and any information to help answer students' questions about the test. Provide remedial training as needed. |

Appendix A - Viewgraph Masters  (N/A)

Appendix B - Test(s) and Test Solution(s)  (N/A)

Appendix C - Practical Exercises and Solutions  (N/A)

Appendix D - Student Handouts  (N/A)