L3 ENVG EVALUATION

| 1. What are the approximate hours of operation with new L91 lithium batteries in fused mode and remaining I2 only mode? |  |
| --- | --- |
|  | Fused 8.2 hours I2 remaining 8 hours |
| x | Fused 7.5 hours I2 remaining 7.5 hours |
|  | Fused 9.3 hours I2 remaining 9.3 hours |
|  | Fused 8.5 hours I2 remaining 7.5 hours |

| 2. Can the LIF and Sacrificial Window be installed together? |  |
| --- | --- |
| x | Yes, LIF under Sacrificial window |
|  | Yes, Sacrificial window under LIF |
|  | No |

| 3. I2 can see through glass. |  |
| --- | --- |
| x | True |
|  | False |
|  | Only on rainy days |
|  | Only if they are looking through Beer Goggles |

| 4. Thermal can see through glass. |  |
| --- | --- |
|  | True |
| x | False |
|  | Only on hot days |
|  | Only on cold days |

| 5. In what type of environment would you install the LIF? |  |
| --- | --- |
|  | In a classroom environment |
|  | In a hostile environment |
| x | In an environment with possible laser threat |
|  | In a target rich environment |

| 6 How can you reduce mirage effect? |  |
| --- | --- |
|  | Calibrate |
|  | Wait for cooler temperature |
|  | Move to higher ground |
| x | Both 2 an 3 |

| 7. In what type of environment would you install Sacrificial window? |  |
| --- | --- |
|  | In Rain/Snow |
|  | Overseas |
| x | Blowing dust and debris/general protection |
|  | In high enemy area |

| 8. What are the 5 modes on the mode selector switch? |  |
| --- | --- |
| x | Off, I2, Thermal, Fused, and Menu |
|  | I2, Thermal, Engagement, Overlay and Menu |
|  | Off, Thermal, Overlay, I2 and Menu |

| 9. What is the highlight cut off shut off time? |  |
| --- | --- |
|  | 30 +/- 30 |
| x | 70 +/- 30 |
|  | 40 +/- 70 |
|  | 20 +/- 100 |

| 10. What is function of IR Illuminator? |  |
| --- | --- |
|  | Turn thermal off in high light conditions |
|  | Provide light source for thermal |
| x | Provide additional light for I2 |
|  | Protect I2 tube from damage |

| 11..What is Diurnal effect? |  |
| --- | --- |
|  | Sunset and Sunrise |
|  | When it’s raining |
| x | 2 times during the day when inanimate objects are close in temperature |
|  | Fall and Spring |

| 12. How do you adjust I2 and thermal brightness? |  |
| --- | --- |
|  | By looking at a bright or dull object |
|  | By double clicking thermal control knob |
|  | With the diopter focus |
| x | By turning either I2 or thermal control knob |

| 13. When using 2 batteries where must they be installed? |  |
| --- | --- |
|  | Innermost |
|  | Outermost |
| x | Inner or outermost |
|  | Anywhere |

| 14. What happens to the batteries in extreme temperatures? |  |
| --- | --- |
|  | Performance increased |
| x | Performance decreased |
|  | No effect |
|  | They don’t work |

| 15. What are the 3 functions of the thermal control knob? |  |
| --- | --- |
|  | Adjust I2 brightness, turn on IR illuminator and calibrate |
|  | Change polarity, calibrate, and turn on highlight cut off |
| x | Adjust thermal brightness, change’s polarity, and calibrates |
|  | Adjust thermal brightness, turn on IR, and turns on highlight cut off |

| 16. What does the diopter focus adjust? |  |
| --- | --- |
|  | The thermal scene |
|  | The I2 Scene |
|  | Both thermal and I2 scene |
| x | The display to the users eye |

| 17. What does the Objective focus adjust? |  |
| --- | --- |
|  | The thermal Scene |
| x | The I2 scene |
|  | The display to the users eye |
|  | Both thermal and I2 scene |

| 18. If in Handheld configuration, can the battery pack be placed on either side? |  |
| --- | --- |
|  | Left side only |
| x | Right side only |
|  | Either |
|  | No handheld use available |

| 19. In fused mode what menu option should be selected for viewing targets beyond 45 meters? |  |
| --- | --- |
|  | Near |
|  | Mid |
| x | Far |
|  | No difference in fused mode |

| 20. What is AGC used for? |  |
| --- | --- |
| x | When viewing scene with low thermal difference’s |
|  | When viewing a scene with high thermal difference’s |
|  | Never |
|  | Outside only |