A.   ITEM TECHNICAL SAFETY INFORMATION:

1.  FSC:  1315

2.  PKG:  1.2.1G			4.  Fire Sym:  2

3.  UNPKG:  1.2.1G		5.  Chem Haz Sym:  N/A

B.  SAFETY PRECAUTIONS:

1.  Uncontrolled discharges of static electricity present a small, but not inconsiderable, ignition danger to propellant charges assembled to the cartridge.  Prior to and during handling and inspection of the propelling charge, care must be taken to dissipate built-up charges of static electricity.

a.  Cartridges will be laid out for inspection on a grounded surface, where possible.

b.  Inspector shall periodically touch the grounding mat or other grounded metal surface (e.g., the grounding lead clipped to the tailgate of a truck, for inspections conducted under fields conditions) in order to dissipate charges that may have built up on his or her person.

2.  A PA154 metal box of two cartridges weighs a nominal 45 kg, or almost 100 lbs.  This requires that two (2) people lift and place a full box.

3.  General safety and handling precautions apply.

C.  INSPECTION PROCEDURES

1.  Outer Pack.  Inspect PA154 metal outer packs for serviceability.  Classify defects IAW SB 742-1.  Sequence of inspection points is immaterial, as long as all inspections are conducted.

| a. Box sealed with antipilferage seal or banding. b. Closing hardware is present, tightly assembled, and not broken. (M) Hardware may be repaired and/or replaced, c. Markings are legible, correct, and complete enough to positively identify the contents. As a minimum, markings must include: (1) C625 (2) Lot number (3) Quantity (4) “Ammunition, illuminating” (5) “UN 0171”. d. Assure PA154 metal boxes are clean, dry, and serviceable. (1) There should be no holes, perforations, split seams, or other breaks. (2) There should be no distortion, creases, or folds in the metal. (3) There should be no dents affecting the structural integrity of the metal box. (4) There should be no dents more than four (4) inches square, nor more than ¼ inch deep. (5) There should be no dents or damage that will preclude taking the items out of the box. (6) Rust or other foreign material, if present, should not obscure markings to the point of precluding identification of the box’s contents. (7) Rust or other foreign material should not interfere with operation of the lid and sealing of the box. (8) Closures should hold the box securely closed. (9) Gasket and sealing surfaces are clean and will provide a watertight fit. (10) Bail handles under lid closures are present and secure. (11) Two (2) handles on top of metal box are present and secure. e. Pack is tight enough to preclude excessive movement of contents. | b. (I) Must be able to securely close the box, and must permit opening of box without having to disassemble box. Repairs or use of banding in lieu of hardware is acceptable. (M) Hardware may be repaired and/or replaced as necessary, but condition requiring repair will be noted on surveillance records. c.(1) (I) See DWG 12577551 for markings. c.(2) (O) Make note of markings which need to be applied to the box(es) prior to shipment; record on the DSR card. UN POP markings are to appear on the outer pack for items manufactured after January 1990. d. (I) Conditions identified at d(1) through (10) are major defects. (10) (I) Missing or loose bail handles are a minor defect. (12) (I) Missing or loose handles are a minor defect. e. (I) Record loose pack as a minor defect, but install enough cushioning material to secure contents prior to repacking rounds. |
| --- | --- |

2.  INNER PACK.  Sequence of inspection points is immaterial, as long as all inspections are conducted.

| a. Assure web lifting assemblies are present and serviceable. b. Assure that web lifting assemblies are secured in place with pressure-sensitive tape in two places on the exterior of the PA153 fiber container. c. Assure PA153 fiber containers are “sealed” with 1-1/4 wraps of pressure-sensitive tape at the junction of the end cap to the container body. d. Assure PA153 fiber containers are clean, dry, and serviceable. (1) There should be no holes, perforations, cuts, tears or gouges. (2) There should be no wrinkles or peeling caused by looseness between layers. (3) There should be no blistering of the surface of the fiber container. (4) There should be no evidence of moisture absorption. (5) There should be no distortion, creases, or folds in the metal ends. (6) There should be no separation of the metal ends from the fiber end cap bodies. (7) There should be no dents or damage that will preclude removal of end caps and/or removal of items from the fiber container. (8) Scuffs or foreign material, if present, should not obscure markings to the point of precluding identification of the box’s contents. (9) Fiber containers are to be marked correctly. Markings must agree with those on the outer pack, and must include: (a) Cartridge, 120mm ILLUM, M930 (b) Lot number b. Assure packing spacers inside the fiber container are present, clean, dry, and serviceable. (1) There is a flat spacer between the base of the fin assembly and the metal of the end cap. (2) There is an insert which secures the fuze and nose of the cartridge in the body of the fiber container. | a. (O) Gently lift the assembly out of the PA154 box, and, with it, the PA153 fiber container. a. (I) Missing assembly is a minor defect as long as the fiber containers can be removed from the PA154. b. (I) Missing tape is a minor defect. b. (Info) If this condition is encountered, investigate the possibility that this is a field return, and not a manufacturing defect. (1) (M) Tears smaller than ½ square inch and cuts which do not penetrate all impregnated layers may be sealed by spot painting IAW TM 9-1300-251-20&P. (2) (I) This condition is irreparable, and is a major defect. (3) (I) This condition is irreparable, and is a major defect. (4) (I) This condition is irreparable, and is a major defect. (5) (I) This condition is irreparable, and is a major (6) (I) This condition is irreparable, and is a major defect. (7) (I) This condition is irreparable, and is a major defect. (2) (Info) The rigid stop is not removable from the fiber container. The shoulders of the cartridge rest upon this stop. (2) (I) Missing support is a major defect. |
| --- | --- |

3.  ITEM.   Sequence of inspection points is immaterial, as long as all inspections are conducted.  Classify defects IAW SB 742-1.

NOTE:  Before proceeding with the inspection, verify that the fuze pin and safety wire assembly is in place, and that the fuze is set on SAFE (“S”).  Fuze missing safety pin/wire and/or not set on “S” is a CRITICAL defect.

a.  Visually, manually inspect the cartridge body to assure that:

| (1) Markings are legible, correct, and match those on the packaging materials. (2) Cartridge body is not bulged, misshapen, or visibly out of round. (3) Cartridge body is free of cracks. (4) Cartridge body is free of rust, adhesives, and/or other foreign material which cannot be removed. (5) Cartridge body paint coverage is complete. (6) Obturating ring is present and whole. (8) Obturating ring is secured in place with adhesives. (9) Obturating band is seated at or below flush with the cartridge body. (10) Obturating band is free from cuts, nicks. (11) Tube/boattail assembly is fully seated against and tightly assembled to the cartridge body. | (1) (I) The illum cartridge is white with black markings. A different marking scheme is a CRITICAL defect. (2) (I) Per TM 9-1300-251-34&P, a visibly dented, distorted cartridge body and/or ogive as a CRITICAL defect. (4) (I) A cracked body is a CRITICAL defect. (5) (I) Foreign material which could interfere with free insertion of the cartridge into the mortar tube is a major defect. (5) (I) Bare spots greater than 0.25 square inch are considered to be minor defects. (6) (I) A missing or broken obturating ring is a CRITICAL defect. NOTE that the diagonal seam in the ring is not a “break”. (7) (I) A loose obturating ring is a major defect. (9) (I) Use a straightedge to verify this condition. A raised obturator is a major defect. (10) (I) A damaged obturator is a major defect. (11) (I) A loose tube/boattail assembly is a major defect. |
| --- | --- |

NOTE:  Do not disassemble fuze from cartridge solely for the purpose of inspection.

b.  Visually, manually inspect the fuze, MTSQ, M776  to assure that:

| (1) Fuze is present, and identification markings are consonant with info on the ADC for the complete round. (2) Safety pin and restraining wire installed and fully engaged. (3) Fuze is set on “S”. (4) Fuze fully and tightly seated against the cartridge ogive. (5) Fuze is not misshapen or otherwise severely damaged; no broken components. (6) Fuze dry and clean, free of foreign material. | (1) (I) A different fuze is a major defect: (2) (I) Missing or disengaged pin/wire assembly is a CRITICAL defect. (2) (M) If the pin cannot be reinserted, contact EOD for disposal. (3) (I) An M776 MTSQ fuze NOT set on “S” is a CRITICAL defect. (3) (M) Fuze may be reset to “S” by turning COUNTERCLOCKWISE until the triangle pointer is directly above the “S”. (4) (I) Fuze must resist disassembly using vigorous hand pressure. If the fuze is not fully and tightly seated, it is a major defect. (5) (I) TM 9-1300-251-34&P classifies a visibly distorted or damaged fuze as a CRITICAL defect. (6) (I) Classification will depend on the extent to which the condition is reparable. |
| --- | --- |

NOTE:  Propellant charge increment “containers” are made of explosive material.  Prior to and during handling of the propelling charge, care MUST be taken to dissipate built-up charges of static electricity.  Inspector shall periodically touch the grounding mat or other grounded metal surface (e.g., the grounding lead clipped to the tailgate of a truck, for inspections conducted under fields conditions) in order to dissipate charges that may have built up on his or her person.

c.  Visually, manually inspect M230-series propelling charge to assure that:

| (1) Propellant charge support/cover assembly in place and properly assembled and secured in place with a bit of tape. (2) Propellant model and lot number match the info on the ADC. (3) All four (4) increments are assembled to the fin and boom assembly. Orientation of increment openings is irrelevant. | (1) (I) Missing or improperly assembled propellant charge support/cover assembly is considered to be a CRITICAL defect. (1) (O) If this condition is encountered, investigate the possibility that the round(s) is/are returned from field use, and that the defect is not a manufacturing defect. Missing assembly is a major defect when found on field returns. (2) A mismatch is a major defect. (3) (I) Missing propellant increments are a CRITICAL defect. (O) If this condition is encountered, investigate the possibility that the round(s) is/are returned from field use, and that the defect is not a manufacturing defect. |
| --- | --- |

NOTE:  Exercise caution not to tear propellant increment containers when removing them from around fin and boom assembly.

| (4) Propellant charge increment containers are not leaking, damaged, or delaminating. (5) Propellant charge increments are clean, dry, and free of foreign material. (6) Model and lot number markings on propellant charge increments match those recorded on ADC. | (4) (I) Leaking and/or damaged propellant charge increment containers are CRITICAL defects. (O) If this condition is encountered, investigate the possibility that the round(s) is/are returned from field use, and that the defect is not a manufacturing defect. (I) Small crushed points or other damage which does NOT permit leaking of propellant is a major defect. (Info) Orientation of the increment openings on the fin and boom assembly is irrelevant. (5) (I) Presence of foreign material which cannot be easily removed by hand is a minor defect. (5) (Info) Adhesion of foam from the support/cover assembly is a known problem with this item. (6) (I) A mismatch is a major defect. |
| --- | --- |

d.  Visually, manually inspect fin and boom assembly to

assure that:

| (1) Fin assembly is not damaged, cracked, or visibly distorted. NOTE: Fins are NOT canted on this round. (2) Fin assembly is free of corrosion and other foreign material. (3) Ignition cartridge present and securely and squarely assembled to the base of the fin assembly. (4) Head of ignition cartridge below flush with the rear edge of the fin assembly. (5) Primer below flush with the head of the ignition cartridge. (6) Ignition cartridge free of corrosion and other foreign material which cannot be removed by wiping. (7) Flash holes in fin and boom assembly clear of material and other blockages. (8) Boom fully seated against and tightly assembled to the tube/boattail assembly of cartridge. (9) Carefully reassemble propellant increments to the fin and boom assembly. | (1) (I) Each of these conditions is a CRITICAL defect. (2) (I) Corrosion and/or metal etching and/or foreign material which cannot be removed with fine sandpaper is a major defect. (3) (I) Missing or improperly assembled ignition cartridge is a major defect. (4) (I) Use a straightedge to verify this condition. A protruding ignition cartridge is a CRITICAL defect. (5) (I) Use a straightedge to verify this condition. A protruding primer is a CRITICAL defect. (6) (I) Foreign material which cannot be removed and which may interfere with firing is a major defect. (7) (I) Any blockage which cannot be easily removed is a major defect. (8) (I) A loose connection is a major defect; a wobbly connection is a CRITICAL defect. (9) (O) Orientation of markings and openings is irrelevant. |
| --- | --- |

4.  REPACKAGING.

NOTE:  Prior to repackaging, assure that the packaging materials bear the correct markings.

| a. Reinstall and close the propellant charge cover/support assembly. b. Place cartridge in its associated end cap, orient it nose end up, and slide the fiber container body over the cartridge and onto the end cap. c. Secure each end cap in place with 1-1/4 counter-clockwise wraps of pressure-sensitive tape. d. Stretch web lifting straps onto fiber containers. e. Set PA154 metal box on end, opening up, and reinstall plywood spacer in “bottom”/far end. f. Reinsert sealed fiber containers into the box, the nose end of each pointing in opposite direction from the other. g. Insert fiberboard spacers as needed to assure a tight pack. h. Close box, secure closing hardware, and apply antipilferage seals to both ends of the lid. | a. (O) Seal in place with two strips of pressure- sensitive tape. Take care not to cover markings. c. (O) Make a tab of approx. one (1) inch at the end of the tape to facilitate opening in the field. f. (O) (M) Exercise care to assure that the web strap lifting assemblies do not slip out of place while inserting the PA 153 fiber containers into the PA154 metal box. |
| --- | --- |

D.  SPECIAL TOOLS AND EQUIPMENT

ITEM					QUANTITY	SPEC/DRAWING	STOCK NUMBER

There are no special tools required for this inspection of the M930 120mm illuminating cartridge.

E.  SUPPLEMENTAL HAZARD ANALYSIS

MISSION: Handle and inspect cartridge, 120mm Illum, M930 w/ fuze, MTSQ, M776; repackage and return following inspection.  Assess hazards peculiar to the inspection of this item.

PREPARED BY:  Mr. Andrew Rayment, QASAS, 401st AFSB, 1 February 2010

TASK			IDENTIFY	ASSESS		CONTROLS		         RESIDUAL

HAZARDS	RISK LEVEL	TO BE IMPLEMENTED		RISK

NOTE:  Ammunition is designed to be safe to ship, store, and handle.  Principle hazards will be those of external forces (e.g., fire or unauthorized handling/disassembly) acting upon the munitions.

| 1. Handle, inspect cartridge 2. Handle, inspect propellant increments | Initiation of round from dropping set fuze Tearing increment containers Static discharge initiates propellant | HIGH (IIC) LOW (IVC) MED (IID) | Personnel selection and training; inclusion of cautionary note in procedure Personnel selection and training; inclusion of cautionary note in procedure Use grounded surface; care to ground self | LOW (IIE) LOW (IVD) LOW (IIE) |
| --- | --- | --- | --- | --- |

OVERALL RISK LEVEL AFTER CONTROLS ARE IMPLEMENTED:  LOW